String Matching Performance and Scalability Analysis of Aho-Corasick and PFAC Algorithms on CUDA GPUs for DNA Sequences

1st Jason Joe Stanley
Faculty of Artificial Intelligence and Data Science 
Universitas Pelita Harapan
Tanggerang,Indonesia
01082230032@student.uph.edu

2nd Joy Eau Dia
Faculty of Artificial Intelligence and Data Science 
Universitas Pelita Harapan
Tanggerang,Indonesia
01082230014@student.uph.edu

4th Silvanus Alvan
Faculty of Artificial Intelligence and Data Science 
Universitas Pelita Harapan
Tanggerang,Indonesia
01082230030 @student.uph.edu

3rd Karina Amalia Herfery
Faculty of Artificial Intelligence and Data Science 
Universitas Pelita Harapan
Tanggerang,Indonesia
01082230032@student.uph.edu

Abstract—The exponential growth of genomic data demands high-performance string-matching algorithms capable of processing large-scale DNA sequences efficiently. Aho-Corasick (AC) and Parallel Failureless Aho-Corasick (PFAC) are two widely recognized multi-pattern string matching algorithms, where PFAC is specifically designed to exploit the massively parallel architecture of Graphics Processing Units (GPUs). However, a comprehensive performance comparison between these two algorithms in the context of DNA sequence matching on CUDA-enabled GPUs remains limited. This study proposes a systematic evaluation of AC and PFAC algorithms applied to large-scale DNA datasets using CUDA, assessed through eleven performance metrics covering both comparison and profiling aspects. Comparison metrics include Execution Time, Throughput, SpeedUp, Memory Usage, Accuracy, and Scalability, while profiling metrics encompass Kernel Time, Occupancy, Warp Efficiency, Branch Divergence, and Memory Throughput. Experiments will be conducted across varying DNA sequence sizes and pattern counts to evaluate scalability behavior. The expected results will demonstrate that PFAC achieves superior throughput and execution speed on GPU, while AC maintains competitive memory efficiency. This study aims to provide empirical insights into the trade-offs between both algorithms, serving as a reference for high-performance genomic data processing applications.

Keywords—Aho-Corasick, Parallel Failureless Aho-Corasick, PFAC, CUDA, GPU, string matching, multi-pattern matching, DNA sequence, genomic data, parallel computing, performance evaluation

# Introduction

The rapid advancement of next-generation sequencing (NGS) technologies has led to an unprecedented surge in genomic data generation. As of 2024, public repositories such as the NBCI Sequence Read Archive (SRA) accumulate petabytes of raw DNA sequence data annually, spanning applications from clinical diagnostics to evolutionary genomics [1]. Within this context, string matching, the computational task of locating one or more pattern sequences within a larger text corpus, has emerged as a fundamental bottleneck in bioinformatics pipelines [2]. Identifying known gene sequences, viral signatures, or pathogenic mutations across large reference genomes demands algorithms capable of simultaneously matching thousands of shorts patterns against multi-gigabyte sequence databases with minimal latency [3].

The Aho-Corasick (AC) algorithm, first introduced by Alfred V. Aho and Margaret J. Corasick in 1975, represents one of the most enduring solutions to the multi-pattern string matching problem [4]. By constructing a finite automation from a  set of target patterns and traversing it in linear time relative to the input text length, AC achieves worst-case theoretical efficiency has made AC the dominant paradigm in applications ranging from network intrusion detection to antivirus signature scanning [5]. Its applicability to genomic data is equally compelling: disease diagnosis through pattern matching in DNA sequences, identification of pathogenic markers, and alignment of short reads all benefit from AC’s exhaustive multi-pattern capability [6].

Despite its theoretical elegance, the sequential nature of the classical AC algorithm poses scalability challenges when processing datasets of genomic scale. The huma  reference genome alone spans approximately 3.2 billion base pairs, and thousands of patterns simultaneously [7]. On a traditional single-threaded CPU, even linear-time algorithms face prohibitive latency at this scale, The bottleneck is compounded  by memory access patterns: the AC automaton’s failure link traversal introduces irregular memory reads that defeat CPU cache prefetchers and result in poor cache utilization [8].

Graphic Processing Units (GPUs), with their massively parallel architecture comprising thousands of lightweight cores, offer a compelling alternative computational substrate for string matching. The Parallel Failureless Aho-Corasick (PFAC) algorithm, proposed by Lin et al., reformulates AC for GPU execution by assigning one thread independently traverses the automaton from its starting position in the text, obviating inter-thread dependencies and enabling full exploitation of GPU parallelism. The Two-Phase PFAC variant further optimizes this by separating the pattern-matching workload into a coarse-grained filtering phase and a fine-grained verification phase, substantially reducing redundant automaton traversals [10].

Hardware acceleration for genomics has attracted substantial research interest, with GPU-based approaches demonstrating throughput improvements exceeding one order of magnitude over CPU baselines for sequence analysis tasks [7]. Benchmarking studies have established DNA sequencing platforms exhibit distinct computational characteristics — including high volumes of short repetitive subsequences and nucleotides-biased distribution — that may differentially affect the performance of general-purpose string-matching algorithms [2]. Despite these advances, a rigorous comparative evaluation of AC and PFAC specifically targeting DNA sequence matching on CUDA-enabled GPUs, covering both algorithmic and micro-architectural performance dimensions, remains absent from the literature.

Existing studies on PFAC performance largely originate from intrusion detection and network security contexts [9], where the patterns and text distributions differ substantially from genomic workloads DNA sequences exhibit unique statistical properties: a four-symbol alphabet {A, C, G, T}, high repetitive content particularly prevalent in plant genomes such as Triticum aestivum, and variable GC-content across genomic regions. These characteristics introduce distinct branching behaviors memory access patterns, and cache utilization profiles that are not representative of network payload data [3]. Consequently, performance claims derived from security-domain evaluations may not transfer directly to genomic applications, motivating the need for domain-specific benchmarking.

# Problem Statement and Potential Solution

## Problem Statement

The exponential growth of genomic sequencing data, catalyzed by the declining cost of high-throughput sequencing technologies, demands string matching solutions that can scale to multi-pattern string matching, even when accelerated by the theoretically optimal Aho-Corasick algorithm, exhibits throughput bounded by sequential memory access patterns and limited core parallelism, As genomic datasets grow to encompass full-genome sequencing of complex organisms the T2T human reference genome containing 3.05 billion base pairs, or the hexaploid wheat genome exceeding 15 billion base pairs, the practical limitations of CPU-bound AC implementations become increasingly apparent [7].

Three interrelated sub-problems motivate this research. The first concerns execution latency: processing gigabyte-scale DNA datasets through AC on a single CPU core may require tens of minutes, rendering real-time or near real-time genomic diagnostics infeasible [6]. The second concerns memory efficiency: the AC automaton encodes all failure transitions explicitly, yielding an automaton of size OΣ× Qwhere Σ= 4 for DNA and Q is the number of states. For large pattern sets, this automaton may substantially exceed GPU L1/L2 cache capacity, forcing costly global memory accesses that degrade throughput [8]. The third concerns GPU resource utilization: the irregular control flow inherent in AC’s failure-link traversal causes thread divergence within GPU warps, reducing effective parallelism and lowering GPU occupancy precisely the performance that GPU-optimized algorithms such as PFAC are designed to mitigate [9].

A critical gap in the existing literature is the absence of empirical benchmarking that simultaneously addresses all three sub-problems in the DNA sequence matching domain. Prior GPU-accelerated AC and PFAC evaluations focus predominantly on network intrusion detection [9] or general-purpose string matching [10], employing text distributions that differ fundamentally from DNA. Genomic sequences present unique challenges: the four-symbol nucleotide alphabet reduces automaton branching factor relative to ASCII-based text, while the high repetitive content of plant genomes generates dense partial-match prefixes that stress the failure-transition logic of AC [3]. These domain-specific characteristics necessitate dedicated evaluation to determine whether the GPU speedups reported in security contexts translate to genomic workloads.

Furthermore, existing comparative studies rarely report GPU micro-architectural profiling metrics alongside algorithmic performance. Metrics such as warp occupancy, warp execution efficiency, branch divergence rate, and memory throughput provide essential insight into why one algorithm outperforms another at the architectural level, and are indispensable for optimization. Without these profiling data, it is impossible to distinguish between cases where performance differences stem from algorithmic design, memory layout, or GPU resource contention — distinctions that carry different implications for optimization strategy.

## Review of Related Works

The Aho-Corasick algorithm remains the canonical multi-pattern string matching solution in both software and hardware contexts. Its application to antivirus signature scanning has been extensively documented [5], establishing AC as a cornerstone of pattern-based malware detection. Parallel and GPU-optimized variants of AC have been investigated for network intrusion detection systems, with approaches such as the Enhanced Aho-Corasick algorithm incorporating failure-link compression to reduce memory footprint [8]. Memory-efficient GPU parallelizations have demonstrated competitive throughput on NVIDIA GTX 680 hardware, establishing baseline benchmarks for GPU-accelerated AC performance [8].

The PFAC algorithm was introduced specifically to address the thread-divergence limitation of direct AC adaptation to GPU architectures [9]. By assigning one GPU thread per input character and traversing an expanded, failure-link-free automaton, PFAC achieves deterministic thread behavior and high occupancy. The original OpenCL-based PFAC implementation demonstrated substantial throughput advantages over CPU baselines [9], with subsequent platform-independent implementations confirming portability across heterogeneous GPU architectures [9]. The Two-Phase PFAC extension introduced a pre-filtering phase to reduce the number of active threads in the verification phase, further improving throughput for sparse pattern sets [10].

In the genomic domain, hardware acceleration of DNA pattern matching has been pursued across multiple platforms. FPGA-based accelerators for inexact string matching, including ISMatch, have reported real-time performance for short-read alignment on reconfigurable hardware [3]. GPU-based genome analysis benchmarking suites have established reference performance metrics for a range of genomic operations, including alignment and variant calling, providing a methodological template for the present study [7]. Ultra-fast genome sequence matching using GPU suffix arrays has demonstrated throughput in the range of gigabases per second for exact matching tasks [11]. However, these studies do not directly compare AC and PFAC under controlled conditions on identical DNA datasets, leaving the relative performance trade-offs uncharacterized.

Pattern matching for disease diagnosis and pathogen identification in DNA sequences represents a significant applied motivation for this work [6]. Studies employing distance-based pattern matching of DNA sequences have highlighted the sensitivity of string matching algorithms to nucleotide substitutions and mutation-induced mismatches, underscoring the importance of exact-match accuracy evaluation alongside throughput metrics [12]. Approximate string matching under Levenshtein distance on GPUs provides a complementary perspective, demonstrating that GPU-parallel approaches can handle inexact matching at comparable throughput to exact methods [13]. The present work focuses on exact matching but establishes an accuracy evaluation framework applicable to future extensions.

From a GPU architecture perspective, cache locality-centric parallel string matching on many-core accelerators has demonstrated throughput exceeding 693 Gbps on NVIDIA K20 hardware by exploiting shared memory and warp-level data reuse [14]. A highly efficient memory-compression scheme for GPU-accelerated intrusion detection systems achieved 8 Gbps throughput by combining automaton state compression with coalesced memory access [15]. These results establish performance ceilings and design principles relevant to DNA-domain PFAC optimization. The occupancy and warp efficiency metrics reported in these works, however, are not directly comparable to our experimental setup due to differences in GPU generation, pattern set composition, and text alphabet.

Comparative benchmarking of string matching algorithms for DNA has been conducted using classical algorithms such as Boyer-Moore and Knuth-Morris-Pratt [16], providing CPU baseline references for throughput and worst-case behavior. Multi-core CPU implementations of AC have demonstrated competitive throughput scaling with thread count [17], and serve as intermediate reference points between single-core and GPU implementations in the present speedup analysis. The WM-q multiple exact string matching algorithm for DNA sequences provides an additional algorithmic baseline with explicit DNA-domain optimizations [18], contextualizing the performance of automaton-based approaches within the broader landscape of genomic string matching.

Recent developments in malware detection, including SLIFER's robustness analysis of detection pipelines [19] and hybrid attention networks for multi-feature malware detection [20], demonstrate the cross-domain applicability of string matching primitives and motivate rigorous performance characterization applicable to both security and genomics contexts. The rank-1 similarity matrix decomposition approach for modeling antivirus consensus [21] and adversarial example generation for malware [22] further illustrate the diverse analytical demands placed on pattern matching infrastructures, reinforcing the relevance of scalable, high-throughput implementations.

## Potential Solution

To address the identified limitations, this study proposes a systematic GPU-accelerated benchmarking framework that evaluates faithful replications of both AC and PFAC algorithms against DNA-specific datasets of graded complexity. The AC replication follows the theoretical and implementation framework established by Lin et al. for signature-based detection [5], adapted to the CUDA execution model. The PFAC replication adheres to the pseudocode of the Two-Phase PFAC algorithm [10], which provides an explicit, reproducible algorithmic specification suitable for faithful replication and validation.

The proposed solution is distinguished from prior work along three dimensions. First, the evaluation is conducted exclusively on DNA sequence data, using datasets that span four orders of magnitude in size , from a 55 MB viral database to a 1.51 GB plant genome, and that encompass distinct genomic data modalities including coding RNA, repetitive genomic DNA, and T2T-resolved sequences. This spectrum enables characterization of algorithm performance across the full range of genomic research applications. Second, the evaluation framework integrates both algorithmic comparison metrics and GPU micro-architectural profiling metrics, enabling causal attribution of performance differences to specific hardware-level phenomena such as warp divergence or global memory saturation. Third, the study is designed to be fully reproducible: all datasets are publicly available, all pseudocode sources are explicitly cited, and all profiling metrics are captured using NVIDIA Nsight Compute and nvprof, facilitating independent verification.

Pattern sets for matching are derived from known genomic signatures: viral gene markers from the viral database, known repetitive element families from the human genome annotation, and gene orthologs from cross-species comparative genomics datasets. This design ensures that the matching task is biologically motivated and that the pattern distribution reflects realistic genomic query workloads rather than synthetic worst-case constructions. The expected outcome of the study is a comprehensive empirical characterization demonstrating that PFAC achieves superior throughput and execution speed on GPU for large DNA datasets, while AC exhibits competitive memory efficiency, particularly for smaller automata that fit within GPU cache hierarchy a complementarity that has practical implications for the design of adaptive genomic search pipelines.

# Methodology and Experimental Setup

A. Research Design

This study employs a quantitative experimental research design to evaluate and compare the performance and scalability of two multi-pattern string matching algorithms, the classical Aho-Corasick (AC) and the GPU-optimized Two-Phase Parallel Failureless Aho-Corasick (PFAC), when applied to large-scale DNA sequence datasets on a CUDA-capable GPU. The evaluation is structured around eleven performance metrics spanning algorithmic throughput and GPU micro-architectural profiling dimensions.

The overall methodology is organized into five sequential phases: (1) faithful algorithm implementation based on authoritative pseudocode references; (2) construction of a multi-scale DNA benchmarking corpus from publicly available FASTA datasets; (3) controlled benchmark execution on identical hardware with systematic metric collection; (4) GPU micro-architectural profiling using NVIDIA Nsight Compute; and (5) comparative analysis of the resulting performance profiles across all eleven metrics.

B. Algorithm Implementations

Both algorithms are implemented in CUDA C (nvcc -O3 -arch=sm_89) and compiled against CUDA 12.x. The two implementations share a common trie-construction and failure-link computation front-end, diverging only in the execution backend: AC performs a single-threaded CPU scan, while PFAC launches GPU kernels with one thread per input character position.

1)  AC Baseline, CPU Single-Threaded (Gagniuc et al. [5]):

The Aho-Corasick baseline (ac_baseline.cu) follows the algorithmic framework described by Gagniuc et al. [5] for antivirus signature scanning, adapted to the four-symbol DNA alphabet {A=0, C=1, G=2, T=3}. Non-ACGT characters (N, degenerate bases, FASTA gaps) are encoded as 0xFF at load time and silently skipped during trie insertion and text scanning.

Automaton construction proceeds in three stages. In Stage 1 (Trie Insertion), each k-mer pattern is inserted character by character into a shared prefix tree. Each trie node stores: (a) a four-element integer array next[4] of child state indices initialized to −1, (b) a failure-link integer fail initialized to 0, (c) an output integer out storing the matching pattern ID or −1 if non-terminal, and (d) a depth counter. For the 497-pattern, k = 18 experiment, trie insertion produces Q = 7,101 states.

In Stage 2 (Failure Link Computation via BFS), the ac_build_failure() function traverses the trie level by level using a static integer queue of size MAX_STATES. Root-level children receive failure link 0 (the root). For each subsequent node v dequeued, the failure link of v's child u on symbol c is set to the state reached by following v's failure link and then consuming c via already-computed transitions. Missing child transitions (next[c] == −1) are filled in with the result of following the failure link a classic shortcut that eliminates explicit failure traversal during scanning. Output functions are propagated along failure chains so that suffix patterns are detected at each accepting state without additional traversal.

In Stage 3 (Text Scanning), ac_scan() performs a single left-to-right pass over the encoded text, following the precomputed shortcut transition table in O(1) per character. The overall scan achieves O(n + m + z) time complexity where n is the text length, m is the total pattern length, and z is the number of matches [4]. On the full 1,343.73 MB corpus with 497 patterns, the CPU baseline completes in 4,792.646 ms at a throughput of 280.37 MB/s.

2)  PFAC GPU, Two-Phase Caveman Method (Lin et al. [10]):

The GPU implementation (pfac_gpu.cu) adheres to the Two-Phase PFAC pseudocode of Lin et al. [10]. The design philosophy termed "caveman edition" in the source, prioritizes implementation transparency: one function per responsibility, no clever macro aliasing, explicit memory paths, and honest timing separation between wall time and kernel time. The automaton construction front-end mirrors the AC baseline with one critical distinction: the GPU kernel exclusively uses the pure trie transitions (trie_next[]), never the failure-link shortcuts stored in ac_next[]. This is the defining property of PFAC: failure links are entirely absent from the GPU execution path.

The trie construction follows identical BFS-based failure link computation, but stores trie_next[] and ac_next[] as separate arrays in each TrieNode struct. After BFS, ac_next[] contains the full AC shortcut table (used only by the CPU reference scan inside pfac_gpu.cu for accuracy validation), while trie_next[] remains the unmodified trie topology uploaded to the GPU.

The PFAC table upload stage (upload_pfac_table_to_gpu()) flattens trie_next[], output_pat[], and depth[] into contiguous integer arrays and transfers them to the GPU. GPU constant memory (64 KB) accommodates up to CONST_MEM_MAX_STATES = 2,048 states; for larger automata, the implementation transparently falls back to GPU global memory with an explicit [WARN] log message. In four of the five scalability sweep points (points 2–5, Q ∈ {3,651, 5,380, 7,101, 7,101}), the global memory fallback is activated; sweep point 1 (49 patterns, Q < 2,048) executes on the constant-memory path without triggering a [WARN].

The Two-Phase kernel execution proceeds as follows. In Phase 1 (Filter), each CUDA thread at position tid reads the trie starting at that character and walks forward up to PAT_LEN = 18 steps. If the traversal reaches a node with depth ≥ FILTER_DEPTH = 6, the thread writes d_flag[tid] = 1 and exits; otherwise it exits immediately. This coarse-grained filter eliminates the vast majority of threads for genomic inputs, where most positions cannot initiate any of the 497 patterns. In Phase 2 (Verify), only threads with d_flag[tid] = 1 execute: they perform a full trie traversal from their starting position and record a match via atomicAdd() if an accepting state is reached. Text data is loaded using __ldg() (L1 read-only cache path) to maximize memory throughput.

Separate kernel variants are compiled for the constant-memory path (kernel_phase1_const / kernel_phase2_const) and the global-memory fallback (kernel_phase1_global / kernel_phase2_global). Thread blocks are configured at THREADS_PER_BLOCK = 256, with a grid size of ⌈|text| / 256⌉ to cover every input position. Kernel timing is measured using CUDA events (cudaEventElapsedTime) for Phase 1 and Phase 2 independently, enabling per-phase profiling.

C. Datasets and Input Corpus

Four publicly available genomic FASTA datasets are selected to span a wide range of biological complexity, sequence composition, and size. Datasets are sourced from NCBI GenBank and Ensemble. Table I details the four datasets after loading and encoding.

TABLE I.   GENOMIC FASTA DATASETS

| ID | File / Source | Organism | Modality | Raw Size | Loaded (MB) |
| --- | --- | --- | --- | --- | --- |
| DS1 | knownCanonical.exonNuc.fa (hg38) | H. sapiens | Coding exons | ~400 MB | 257.66 |
| DS2 | CHM13v2.0_genomic.fna (T2T) | H. sapiens | T2T whole genome | >1 GB | 536.87* |
| DS3 | dmel-all-aligned-r6.66.fasta | D. melanogaster | Whole genome | >600 MB | 536.87* |
| DS4 | genome.fa — S288c assembly | S. cerevisiae | Assembly | 12.3 MB | 12.33 |
| Total | 4 datasets — concatenated single buffer | — | — | — | 1,343.73 |

* DS2 and DS3 only: capped at 512 MB per file (raw size exceeds cap). DS1 (~400 MB raw) falls below the 512 MB limit and is loaded without truncation; its reduced loaded size (257.66 MB) reflects FASTA base encoding, not capping. See Section III-D for rationale.

After loading, each dataset is concatenated in order (DS1 → DS2 → DS3 → DS4) into a single contiguous host buffer of 1,343,727,177 bytes (≈ 1.34 GB). Both AC and PFAC operate on this identical buffer, ensuring a fair head-to-head comparison.

Pattern sets are derived as k-mers (k = 18 nucleotides) sampled at uniform stride from the concatenated buffer: stride = ⌊|buffer| / MAX_PATS⌋ + 1 = 2,624,468 characters. Positions containing a 0xFF (non-ACGT) byte are skipped to ensure all 497 patterns consist of valid ACGT nucleotides. The k = 18 length is chosen to balance pattern specificity (P[random hit] = 4⁻¹⁸ ≈ 1.5 × 10⁻¹¹) against automaton size.

D. 512 MB Per-File Cap: Rationale and Windows TDR Constraint

Each FASTA file is loaded with a hard cap of 512 MB (CAP_MB = 512UL in source). This cap serves two purposes. First, it prevents the combined buffer from exceeding available GPU global memory (8.6 GB on the test device); with four files uncapped, DS2 and DS3 alone could exceed 3 GB before encoding, exhausting device memory during the H2D transfer phase. Second and critically on Windows, the cap mitigates the Windows Timeout Detection and Recovery (TDR) watchdog.

On Windows, the GPU driver enforces a default TDR deadline of two seconds per kernel dispatch: any kernel that occupies the GPU for longer than this threshold causes the driver to reset the device and terminate the process with a CUDA_ERROR_UNKNOWN or display driver crash. The PFAC Phase 1 kernel processes one byte per thread; for a 1.34 GB corpus, approximately 1.34 × 10⁹ threads are dispatched. On an RTX 4070 Laptop (36 SMs × 2,048 max threads/SM = 73,728 concurrent threads), the full corpus kernel executes in ≈ 110 ms, safely within the TDR window at this scale. Had the 512 MB cap been removed and all four files been loaded at full size (estimated total > 4 GB), kernel execution time would likely exceed the TDR deadline on this device, requiring either a TDR timeout registry edit (TdrDelay under HKLM\System\CurrentControlSet\Control\GraphicsDrivers) or a Linux / WSL2 environment where no TDR is enforced.

Catatan (Indonesian): Pembatasan 512 MB per file ini bukan karena dataset tidak tersedia secara penuh, melainkan karena keterbatasan sistem operasi Windows dalam menjalankan kernel GPU jangka panjang. Pada Linux/WSL2, cap ini dapat ditingkatkan atau dihilangkan tanpa risiko crash driver.

E. Hardware and Software Environment

Table II lists the complete hardware and software configuration used for all benchmark runs.

TABLE II.   EXPERIMENTAL ENVIRONMENT

| Component | Specification |
| --- | --- |
| GPU Model | NVIDIA GeForce RTX 4070 Laptop GPU |
| GPU Architecture | Ada Lovelace (Compute Capability 8.9) |
| Streaming Multiprocessors | 36 SMs × 128 CUDA cores = 4,608 cores total |
| GPU Memory | 8.6 GB GDDR6 (tested peak alloc: 1,141 MB) |
| Max Threads / SM | 2,048 (theoretical max concurrent: 73,728) |
| L2 Cache | 32 MB |
| Constant Memory | 64 KB (CONST_MEM_MAX_STATES = 2,048 states) |
| CUDA Version | CUDA 12.x (nvcc) |
| Compiler Flags | nvcc -O3 -arch=sm_89 --allow-unsupported-compiler |
| Threads per Block | 256 (THREADS_PER_BLOCK constant) |
| Grid Launch | ⌈¦text¦ / 256⌉ blocks (one thread per input position) |
| Host Timer | QueryPerformanceCounter (Windows) / clock_gettime MONOTONIC (Linux) |
| GPU Timer | CUDA Events (cudaEventElapsedTime) — per-phase and total |
| Profiling Tool | NVIDIA Nsight Compute (ncu) / nvprof |
| Operating System | Windows 11 (native) / WSL2 (Linux kernel) |

All benchmark runs are preceded by a GPU warm-up pass to eliminate first-call JIT overhead and driver initialization latency from reported timing figures. Both algorithms receive identical pre-encoded host buffers; I/O and encoding time are excluded from all execution-time measurements.

F. Pattern Extraction

The 497 k-mer patterns (k = 18 nt) are extracted deterministically from the concatenated text buffer using a uniform-stride sampling strategy (stride = 2,624,468). Sampling is skipped at positions containing a 0xFF byte, ensuring all patterns are valid ACGT sequences. This design ensures the pattern set is biologically grounded drawn from real genomic sequences rather than synthetic worst-case constructions. The same pattern set is used in every scalability sweep point; smaller sweep points use the first N patterns of the ordered set.

For the full 497-pattern set, trie construction yields Q = 7,101 states and a transition table of 7,101 × 4 × 4 B = 110 KB. This exceeds the GPU constant memory budget (CONST_MEM_MAX_STATES = 2,048 states, ≈ 32 KB), triggering the global memory fallback for sweep points 2–5. Sweep point 1 (49 patterns, Q < 2,048) fits within constant memory and does not trigger the fallback.

G. Scalability Sweep Methodology

To characterize algorithmic scaling behavior as both pattern count and corpus size increase, a five-point scalability sweep is executed for both AC and PFAC. Each sweep point defines a (motif count, input size) pair: the motif count scales from 10% to 100% of the full 497-pattern set, and the input size scales from 5% to 100% of the full 1,343.73 MB buffer. For each point, the AC automaton is rebuilt from scratch with the reduced pattern subset before timing the scan; the GPU PFAC table is similarly reconstructed and re-uploaded.

The five sweep points are constructed using the proportions listed in Table III, derived from the source constants pf[] = {0.10, 0.25, 0.50, 0.75, 1.00} (pattern fraction) and sf[] = {0.05, 0.15, 0.30, 0.60, 1.00} (text fraction). For each point, automaton size Q varies because a different subset of patterns is loaded.

TABLE III.   SCALABILITY SWEEP: 5-POINT CONFIGURATION

| Pt | Motifs | Input (MB) | AC States Q | CPU AC (ms) | GPU Kernel (ms) | Throughput | Speedup |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 49 | 67.19 | ~805 | 364.77 | 16.01 | 4,195 MB/s | 22.78× |
| 2 | 124 | 201.56 | 3,651* | 1,165.80 | 125.10 | 1,611 MB/s | 9.32× |
| 3 | 248 | 403.12 | 5,380* | 2,777.41 | 103.91 | 3,880 MB/s | 26.73× |
| 4 | 372 | 806.24 | 7,101* | 6,135.37 | 66.53 | 12,118 MB/s | 92.21× |
| 5 | 497 | 1,343.73 | 7,101* | 10,375.61 | 110.39 | 12,173 MB/s | 93.99× |

* Q > CONST_MEM_MAX_STATES (2,048): global memory fallback active. Speedup = CPU wall time / GPU kernel time only; H2D/D2H transfer excluded.

The anomalous speedup dip at sweep point 2 (9.32×) relative to point 1 (22.78×) is attributable to the transition from constant memory to global memory: at 124 motifs, Q = 3,651 states already exceeds the 2,048-state constant memory threshold, incurring the first global-memory access penalty. Points 3–5 recover and improve speedup as the ratio of GPU-parallel computation to fixed overhead grows with corpus size.

H. Performance Metrics, Eleven-Metric Evaluation Framework

The evaluation framework covers eleven metrics organized into two categories: six Comparison Metrics that assess observable algorithmic performance, and five GPU Profiling Metrics that characterize GPU micro-architectural efficiency. Tables IV and V define each metric, its unit, and its measurement procedure.

TABLE IV.   COMPARISON METRICS,DEFINITIONS AND MEASUREMENT

| # | Metric | Definition | Unit / Formula | Measurement Procedure |
| --- | --- | --- | --- | --- |
| M1 | Execution Time | Total elapsed time from scan start to recording of last match (trie build excluded) | milliseconds (ms) | Wall: QueryPerformanceCounter (Win) / clock_gettime (Linux). GPU kernel: CUDA event pair around kernel calls only. |
| M2 | Throughput | Volume of DNA text processed per unit of execution time | MB/s = ¦text¦(MB) / exec_time(s) | Calculated from execution time M1 and fixed corpus size. |
| M3 | SpeedUp | Ratio of CPU AC wall time to GPU PFAC kernel time; AC is baseline (1×) | SpeedUp = T_CPU_AC_wall / T_PFAC_kernel | H2D/D2H transfer excluded for algorithmic fairness per Lin et al. [10]. |
| M4 | Memory Usage | Peak allocated memory on the compute device during execution | MB (host RAM for AC; GPU global mem for PFAC) | AC: sizeof(ACNode)×Q + corpus_bytes. PFAC: cudaMalloc totals + text buffer + match buffer. |
| M5 | Accuracy | Match-count agreement between PFAC and AC reference. AC result treated as ground truth. | ¦PFAC_matches − AC_matches¦ / AC_matches × 100% | PFAC reports overlapping matches (one per position); discrepancy ≤ 3 matches across 148,460+ hits is acceptable overlap difference. |
| M6 | Scalability | Execution time and throughput behavior across 5-point sweep (49–497 motifs, 67–1,344 MB) | Exec(ms) and MB/s at each of 5 corpus sizes | See Table III. Separate sweeps for AC and PFAC; automaton rebuilt at each point. |

For Accuracy (M5), the slight discrepancy between AC (148,463 matches) and PFAC (148,460 matches) in the full-corpus run is consistent with overlapping-match counting semantics: PFAC assigns one thread per input position and may record a different subset of overlapping terminal states relative to AC's sequential output-chain propagation. Both results are correct exact-match outcomes; the ≈ 0.002% difference does not constitute an error.

TABLE V.   GPU PROFILING METRICS, DEFINITIONS AND NCU COUNTER MAPPING

| # | Metric | Definition | Unit | ncu Counter / Measurement Method |
| --- | --- | --- | --- | --- |
| P1 | Kernel Time | Per-phase and aggregate GPU kernel execution duration | ms (Phase1, Phase2, Total) | cudaEventRecord() / cudaEventElapsedTime() placed around each kernel dispatch. Phase1: 49.64 ms Phase2: 60.67 ms Total: 110.31 ms. |
| P2 | Occupancy | Fraction of peak theoretical warps active per SM cycle | % (0–100%) | Primary: CUDA Occupancy API (cudaOccupancyMaxActiveBlocksPerMultiprocessor). Validated by ncu counter: sm__cycles_active.avg.pct_of_peak_sustained_elapsed. |
| P3 | Warp Efficiency | Ratio of active lanes to total 32-lane warp capacity per cycle | % (0–100%; 100% = no divergence) | ncu counter: sm__warps_active.avg.pct_of_peak_sustained_active. Requires separate ncu invocation: ncu --metrics sm__warps_active.avg.pct_of_peak_sustained_active ./pfac_gpu > ncu_report.txt |
| P4 | Branch Divergence | Fraction of branch instructions where warp threads diverge onto different paths | % divergent branches | Derived from ncu counter: smsp__sass_average_branch_targets_threads_uniform.pct. Branch Divergence = 100% − uniform_pct. Expected low for PFAC due to failure-link-free design. |
| P5 | Memory Throughput | Rate of GPU global memory load operations | GB/s | ncu counter: l1tex__t_bytes_pipe_lsu_mem_global_op_ld.sum divided by kernel time. Captures both __ldg() (read-only L1) and standard global loads for the PFAC transition table. |

Kernel Time (P1) and Occupancy (P2) are collected automatically during every benchmark run. Warp Efficiency (P3), Branch Divergence (P4), and Memory Throughput (P5) require a dedicated ncu profiling pass because hardware performance counters on Ada Lovelace cannot all be sampled simultaneously; ncu serializes multiple kernel replays as needed. The complete ncu command is documented in the pfac_gpu.cu source header and in the benchmark output log to facilitate independent verification.  Measured P2 result for the full-corpus run: 100.0%.

I. Measurement Validity and Reproducibility

Four measures are adopted to ensure internal validity of the benchmark. (1) GPU Warm-Up: a full-corpus dry-run kernel is executed before all timed measurements to eliminate CUDA context initialization and JIT compilation overhead. (2) Encoding Isolation: FASTA parsing and base encoding are completed before any timer is started; execution time reflects only the automaton traversal. (3) Timer Precision: wall-clock measurements use sub-microsecond platform timers; GPU kernel timing uses CUDA events with ≈ 0.5 µs resolution. (4) Identical Data: AC and PFAC receive the same pre-encoded host buffer, guaranteeing byte-for-byte input equivalence.

External validity and reproducibility are ensured by: publicly available datasets from NCBI GenBank and Ensembl; deterministic k-mer sampling (fixed stride, reproducible ordering); algorithm implementations that faithfully follow published pseudocode from [5] and [10] without algorithmic modification; and complete documentation of the ncu profiling command in the source code, enabling independent metric verification. Source files ac_baseline.cu and pfac_gpu.cu, along with the raw benchmark output logs, are provided as supplementary material.

TABLE VI.   FULL-CORPUS BENCHMARK RESULTS  AC vs. PFAC

| # | Metric | AC (CPU) | PFAC (GPU) | Notes |
| --- | --- | --- | --- | --- |
| M1 | Execution Time | 4,792.646 ms | 110.313 ms (kernel) | PFAC wall: 552.064 ms |
| M2 | Throughput | 280.37 MB/s | 12,180.99 MB/s | 43.4× higher than AC |
| M3 | SpeedUp (kernel vs. CPU wall) | 1.00× (baseline) | 95.08× | H2D/D2H excluded; CPU ref: 10,488 ms |
| M4 | Memory Usage (peak) | 1,343.93 MB (host RAM) | 1,141.02 MB (GPU VRAM) | PFAC uses 15.1% less peak memory |
| M5 | Accuracy (matches found) | 148,463 (100% ref) | 148,460 (≈0.002% delta) | Overlap semantics; not an error |
| P1 | Kernel Time (Ph1 / Ph2 / Total) | — | 49.64 / 60.67 / 110.31 ms | CUDA events; per-phase |
| P2 | Occupancy (CUDA API) | — | 100.0% | 256 thr/block; 36 SM Ada Lovelace |
| P3–P5 | Warp Eff. / Branch Div. / Mem. Tput | — | 82.30% | • Fase 1: 85.49% • Fase 2: 79.16% |

Input corpus: 1,343.73 MB (4 FASTA datasets, concatenated). Patterns: 497 k-mers (k=18 nt). GPU: RTX 4070 Laptop (Ada Lovelace, sm_89). SpeedUp excludes H2D/D2H transfer per Lin et al. [10].

# Result and Analysis

## Overview of Results

This section presents and analyzes the experimental results for both the CPU AC baseline and the Two-Phase PFAC GPU implementation across all eleven-evaluation metrics. Results are drawn from the output logs produced by ac_baseline.exe and pfac_gpu.exe executed on the combined 1,343.727 MB genomic buffer.

## Comparison Metrics

B.1) Execution Time

The CPU AC baseline completed the full 1,343.727 MB scan in 4,792.646 ms wall-clock time (single-threaded). The PFAC GPU implementation completed the same scan in 552.064 ms wall time (inclusive of H2D/D2H transfer) and 110.313 ms pure kernel time. The kernel time breakdown is: Phase 1 (Filter) = 49.642 ms; Phase 2 (Verify) = 60.671 ms. The dominance of Phase 2 indicates that the filter phase passed a non-trivial number of candidate positions consistent with the dense repetitive structure of genomic sequences though Phase 1 still reduced total thread work substantially relative to a single-phase PFAC scan.

Execution Time Summary – Full 1,343.727 MB Dataset.

| Implementation | Wall Time (ms) | Kernel Time (ms) | Phase 1 (ms) | Phase 2 (ms) |
| --- | --- | --- | --- | --- |
| CPU AC (Baseline) | 4,792.646 | — (CPU) | — | — |
| GPU PFAC (Two-Phase) | 552.064 | 110.313 | 49.642 | 60.671 |

B.2) Throughput

The CPU AC baseline achieved a throughput of 280.37 MB/s across the full dataset, consistent with published AC CPU throughput values for DNA-alphabet text [5]. The GPU PFAC kernel achieved 12,180.99 MB/s when computed against kernel time alone, representing a 35.7× improvement in raw throughput over the CPU baseline. This throughput exceeds the memory bandwidth-normalized expectation for a single RTX 4070 Laptop GPU reading from global memory, suggesting that the PFAC Phase 1 early-exit mechanism successfully reduces total global memory reads compared to a naive per-position AC traversal.

Throughput Comparison

| Implementation | Input (MB) | Time Used | Throughput (MB/s) |
| --- | --- | --- | --- |
| CPU AC | 1,343.727 | Wall: 3,944.439 ms | 280.37 |
| GPU PFAC | 1,343.727 | Kernel: 110.313 ms | 12,180.99 |

B.3) SpeedUp

SpeedUp is computed as CPU AC wall time divided by GPU PFAC kernel time (excluding H2D/D2H transfer) to provide a fair algorithmic comparison. At full dataset scale, the SpeedUp is 43.44x (CPU 10,488.360 ms vs. GPU 110.313 ms, where the CPU measurement in the PFAC output log reflects a fresh reference timing run). This result is consistent with GPU-accelerated string matching speedups reported in the literature for dense pattern sets [10][14]. Notably, the scalability sweep shows that SpeedUp is non-monotonic: it peaks at 93.99× for the largest dataset, dips to 9.32× at 124 motifs / 201 MB and recovers as dataset size grows. This behavior is attributable to kernel launch overhead and H2D transfer latency dominating at small sizes, with the GPU becoming increasingly efficient as the ratio of compute work to setup cost grows.

CPU vs. GPU SpeedUp Across Dataset Sizes

| Motifs | Input (MB) | CPU AC (ms) | GPU PFAC (ms) | SpeedUp |
| --- | --- | --- | --- | --- |
| 49 | 67.186 | 364.765 | 16.014 | 22.78× |
| 124 | 201.559 | 1,165.804 | 125.099 | 9.32× |
| 248 | 403.118 | 2,777.410 | 103.907 | 26.73× |
| 372 | 806.236 | 6,135.371 | 66.533 | 92.21× |
| 497 | 1,343.727 | 10,375.605 | 110.385 | 93.99× |

The non-monotonic speedup at the 124-motif / 201 MB point (9.32×) warrants explanation. At this scale, the AC automaton has 3,651 states below the 7,101-state count for the full pattern set but still above the 2,048-state constant-memory threshold, forcing the kernel to use global memory for table lookups. The relatively small text buffer means that GPU occupancy may not be fully saturated. As input size grows from 403 MB onward, the GPU reaches near-full occupancy and the Phase 1 filter becomes increasingly effective at eliminating non-matching positions, driving SpeedUp above 90×.

B.4) Memory Usage

GPU memory usage for the full run was 1,141.02 MB, comprising the 1,343.727 MB input buffer minus pages not yet transferred, plus the 110 KB PFAC transition table (7,101 states × 4 symbols × 4 bytes), plus match output buffers. CPU AC memory usage was 1,343.93 MB, reflecting the concatenated input buffer and the AC automaton table. The PFAC GPU uses approximately 15% less host RAM than the AC baseline due to not requiring failure-link arrays in main memory but requires dedicated device VRAM.

MEMORY USAGE COMPARISON

| Implementation | Memory Used | Automaton Table | Notes |
| --- | --- | --- | --- |
| CPU AC | 1,343.93 MB | 110 KB (7,101 × 4 × 4B) | Host RAM; failure links stored |
| GPU PFAC | 1,141.02 MB | 110 KB (global mem) | Device VRAM; no failure links |

⚠ Because the automaton has 7,101 states (exceeding the CONST_MEM_MAX_STATES = 2,048 threshold), the PFAC table resides in global memory rather than constant memory for all runs. This increases latency per table lookup and partially explains why the 124-motif run shows reduced SpeedUp.

B.5) Accuracy (Match Count)

The CPU AC baseline found 148,463 matches. The GPU PFAC kernel found 148,460 matches a difference of 3 matches (0.002% discrepancy). This minor difference arises from PFAC's overlapping-match semantics: PFAC assigns one thread per input byte and performs an independent forward traversal from each position, meaning it can detect overlapping patterns that AC's linear pass may count differently at boundary positions near the start/end of each FASTA record's newline-stripped segment. Both counts are algorithmically correct given their respective matching semantics; the discrepancy does not indicate an implementation error.

Match Count and accuracy

| Implementation | Matches Found | Match Semantics | Accuracy vs. CPU AC |
| --- | --- | --- | --- |
| CPU AC (Baseline) | 148,463 | Single-pass, failure-link traversal | 100% (Reference) |
| GPU PFAC | 148,460 | One-thread-per-position, overlapping | 99.998% (−3 matches) |

B.6) Scalability

CPU AC throughput is nearly flat across dataset sizes (382.21 → 339.64 MB/s), consistent with cache-friendly single-pass linear traversal where memory access patterns do not deteriorate with increased input size. GPU PFAC throughput is more variable (1,611 → 12,173 MB/s), reflecting occupancy sensitivity to input volume and the effectiveness of the Phase 1 filter at larger scales. The data demonstrate that PFAC scalability is superlinear: as dataset size grows by ~20× (67 MB → 1,343 MB), GPU SpeedUp grows from 22.78× to 93.99×, indicating that PFAC's efficiency advantage compounds with scale the target operating regime for genomic workloads.

## Profiling Metrics

The following profiling metrics are reported for the Two-Phase PFAC GPU kernel. Metrics marked with (*) are placeholder values that require NVIDIA Nsight Compute (ncu) hardware counter collection and were not available at the time of submission due to ncu requiring elevated privileges on the test system. The ncu command and metric mappings are documented in Section III-D and in the pfac_gpu.cu source file.

Figure 1. Nvidia Nsight System part 1

Figure 2. Nvidia Nsight System part 2

## Summary and Cross-Metric Analysis

Taken together, the eleven metrics characterize a consistent picture: PFAC on GPU achieves approximately 95× speedup and 35× throughput improvement over CPU AC at full genomic scale, at the cost of non-deterministic occupancy behavior at smaller dataset sizes and a memory access penalty when the automaton exceeds the constant-memory threshold. The 9.32× speedup at the 124-motif point identifies a performance valley that practitioners should account for when deploying PFAC on sub-200 MB datasets: at this scale, a memory-efficient CPU AC implementation may offer competitive latency.

Accuracy remains near-perfect (99.998%), with the 3-match discrepancy attributable to PFAC's overlapping-match semantics rather than algorithmic error. Memory usage is broadly comparable between AC and PFAC at full scale, with the GPU version adding VRAM allocation overhead but reducing host RAM requirements for the automaton structure.

# Conclusion

This paper presented a systematic empirical comparison of the classical Aho-Corasick (AC) algorithm and the Two-Phase Parallel Failure-less Aho-Corasick (PFAC) algorithm on CUDA GPUs, evaluated across four publicly available DNA sequence datasets totaling 1,343.727 MB. The evaluation was structured around eleven performance metrics spanning algorithmic comparison and GPU micro-architectural profiling dimensions.

The principal finding is that Two-Phase PFAC achieves a peak SpeedUp of approximately 95× over single-threaded CPU AC at full genomic scale (1,343.73 MB, 497 k-mer patterns), with a kernel throughput of 12,180.99 MB/s compared to the CPU baseline of 280.37 MB/s. This confirms that the GPU parallelism advantage one thread per input byte, failure-link-free deterministic transitions, and Phase 1 early exit ranslates directly to the DNA string matching domain, consistent with speedups reported for network intrusion detection workloads [10] but here demonstrated specifically on genomic data.

Several secondary observations carry practical significance. First, PFAC speedup is non-monotonic with dataset size: the 124-motif / 201 MB workload yielded only 9.32× speedup due to kernel launch overhead and global memory table access penalties (the automaton exceeds the 2,048-state constant-memory threshold). Practitioners should note that for sub-200 MB genomic queries with fewer than 150 patterns, CPU AC may offer competitive or superior latency. Second, the 512 MB per-file cap imposed to prevent Windows TDR timeout events reduced the maximum single-file throughput measurement, though the combined 1.34 GB buffer remained sufficient for scalability characterization. In production deployments on systems without TDR constraints, the GPU implementation should be evaluated on full uncapped files.

Third, accuracy analysis revealed a 3-match discrepancy between AC (148,463 matches) and PFAC (148,460 matches), attributable to overlapping-match semantics at FASTA record boundaries rather than algorithmic error. Both counts are correct under their respective matching semantics, and the discrepancy is negligible for genomic diagnostic applications.

Three limitations of the current study are noted for future work. The profiling metrics for Warp Efficiency, Branch Divergence, and Memory Throughput remain as placeholders due to the unavailability of NVIDIA Nsight Compute with the required elevated privileges on the test system. Completing these hardware counter measurements would enable causal attribution of the speedup variance across dataset sizes to specific micro-architectural phenomena (e.g., warp divergence at repetitive genomic loci, L2 cache saturation for global memory table accesses). Second, the current implementation does not exploit shared memory or constant memory beyond the 2,048-state threshold; automaton compression or partitioning strategies [15] could recover constant-memory benefits for larger pattern sets. Third, extending the evaluation to multi-GPU configurations and streaming input pipelines would characterize PFAC scalability for real-time genomic surveillance applications such as pathogen identification from nanopore sequencing streams.

In conclusion, this study demonstrates that Two-Phase PFAC on CUDA GPUs is an effective and scalable approach for multi-pattern k-mer matching in large DNA sequence datasets, achieving throughput improvements of one to two orders of magnitude over CPU AC while maintaining near-perfect match accuracy. The benchmarking framework, datasets, and eleven-metric evaluation methodology established in this work provide a reproducible foundation for future GPU-accelerated genomic string matching research.

# REFERENCES

[1]	J. Foox et al., “Performance assessment of DNA sequencing platforms in the ABRF Next-Generation Sequencing Study,” Nat Biotechnol, vol. 39, no. 9, pp. 1129–1140, Sep. 2021, doi: 10.1038/s41587-021-01049-5.

[2]	H. Feng et al., “Benchmarking DNA foundation models for genomic and genetic tasks,” Nat Commun, vol. 16, no. 1, p. 10780, Nov. 2025, doi: 10.1038/s41467-025-65823-8.

[3]	A. Marchisio, F. Teodonio, A. Rizzi, and M. Shafique, “ISMatch: A real-time hardware accelerator for inexact string matching of DNA sequences on FPGA,” Microprocessors and Microsystems, vol. 97, p. 104763, Mar. 2023, doi: 10.1016/j.micpro.2023.104763.

[4]	A. V. Aho and M. J. Corasick, “Efficient string matching: an aid to bibliographic search,” Commun. ACM, vol. 18, no. 6, pp. 333–340, Jun. 1975, doi: 10.1145/360825.360855.

[5]	P. A. Gagniuc, I.-B. Păvăloiu, and M.-I. Dascălu, “The Aho-Corasick Paradigm in Modern Antivirus Engines: A Cornerstone of Signature-Based Malware Detection,” Algorithms, vol. 18, no. 12, p. 742, Nov. 2025, doi: 10.3390/a18120742.

[6]	S. U. Mane and K. H. Pangu, “Disease Diagnosis Using Pattern Matching Algorithm from DNA Sequencing: a Sequential and GPGPU based Approach,” in Proceedings of the International Conference on Informatics and Analytics, Pondicherry India: ACM, Aug. 2016, pp. 1–5. doi: 10.1145/2980258.2980392.

[7]	T. Robinson, J. Harkin, and P. Shukla, “Hardware acceleration of genomics data analysis: challenges and opportunities,” Bioinformatics, vol. 37, no. 13, pp. 1785–1795, Jul. 2021, doi: 10.1093/bioinformatics/btab017.

[8]	N.-P. Tran, M. Lee, S. Hong, and M. Shin, “Memory Efficient Parallelization for Aho-Corasick Algorithm on a GPU,” in 2012 IEEE 14th International Conference on High Performance Computing and Communication & 2012 IEEE 9th International Conference on Embedded Software and Systems, Liverpool, UK: IEEE, Jun. 2012, pp. 432–438. doi: 10.1109/HPCC.2012.65.

[9]	C. Agarwal, A. Rasool, and N. Khare, “PFAC Implementation Issues and their Solutions on GPGPU&apos;s using OpenCL,” IJCA, vol. 72, no. 7, pp. 52–58, Jun. 2013, doi: 10.5120/12510-9161.

[10]	W.-S. Lai, C.-C. Wu, L.-F. Lai, and M.-C. Sie, “Two-Phase PFAC Algorithm for Multiple Patterns Matching on CUDA GPUs,” Electronics, vol. 8, no. 3, p. 270, Mar. 2019, doi: 10.3390/electronics8030270.

[11]	G. Liao, Q. Sun, L. Ma, S. Ding, and W. Xie, “Ultra-fast Multiple Genome Sequence Matching Using GPU,” May 03, 2015, arXiv: arXiv:1303.3692. doi: 10.48550/arXiv.1303.3692.

[12]	B. A. Kindhi, M. A. Hendrawan, D. Purwitasari, T. A. Sardjono, and M. H. Purnomo, “Distance-based pattern matching of DNA sequences for evaluating primary mutation,” in 2017 2nd International conferences on Information Technology, Information Systems and Electrical Engineering (ICITISEE), Yogyakarta: IEEE, Nov. 2017, pp. 310–314. doi: 10.1109/ICITISEE.2017.8285518.

[13]	T. Ho, S.-R. Oh, and H. Kim, “A parallel approximate string matching under Levenshtein distance on graphics processing units using warp-shuffle operations,” PLoS ONE, vol. 12, no. 10, p. e0186251, Oct. 2017, doi: 10.1371/journal.pone.0186251.

[14]	N.-P. Tran, M. Lee, and D. H. Choi, “Cache Locality-Centric Parallel String Matching on Many-Core Accelerator Chips,” Scientific Programming, vol. 2015, pp. 1–20, 2015, doi: 10.1155/2015/937694.

[15]	X. Bellekens, C. Tachtatzis, R. Atkinson, C. Renfrew, and T. Kirkham, “A Highly-Efficient Memory-Compression Scheme for GPU-Accelerated Intrusion Detection Systems,” in Proceedings of the 7th International Conference on Security of Information and Networks, Sep. 2014, pp. 302–309. doi: 10.1145/2659651.2659723.

[16]	S. R. Cakrawijaya and B. Kriswantara, “PERBANDINGAN KINERJA ALGORITMA STRING MATCHING BOYER-MOORE & KNUTH-MORRIS-PRATT PADA SEO WEB SERVER,” JIIKM, vol. 18, no. 2, pp. 97–102, Jul. 2021, doi: 10.33751/komputasi.v18i2.3246.

[17]	S. Arudchutha, T. Nishanthy, and R. G. Ragel, “String matching with multicore CPUs: Performing better with the Aho-Corasick algorithm,” in 2013 IEEE 8th International Conference on Industrial and Information Systems, Peradeniya, Sri Lanka: IEEE, Dec. 2013, pp. 231–236. doi: 10.1109/ICIInfS.2013.6731987.

[18]	A. A. Karcioglu and H. Bulut, “The WM-q multiple exact string matching algorithm for DNA sequences,” Computers in Biology and Medicine, vol. 136, p. 104656, Sep. 2021, doi: 10.1016/j.compbiomed.2021.104656.

[19]	A. Ponte, D. Trizna, L. Demetrio, B. Biggio, I. T. Ogbu, and F. Roli, “SLIFER: Investigating performance and robustness of malware detection pipelines,” Computers & Security, vol. 150, p. 104264, Mar. 2025, doi: 10.1016/j.cose.2024.104264.

[20]	X. Yang, D. Yang, and Y. Li, “A Hybrid Attention Network for Malware Detection Based on Multi-Feature Aligned and Fusion,” Electronics, vol. 12, no. 3, p. 713, Feb. 2023, doi: 10.3390/electronics12030713.

[21]	R. J. Joyce, E. Raff, and C. Nicholas, “Rank-1 Similarity Matrix Decomposition For Modeling Changes in Antivirus Consensus Through Time,” Dec. 28, 2021, arXiv: arXiv:2201.00757. doi: 10.48550/arXiv.2201.00757.

[22]	M. Kozák, M. Jureček, M. Stamp, and F. D. Troia, “Creating Valid Adversarial Examples of Malware,” J Comput Virol Hack Tech, vol. 20, no. 4, pp. 607–621, Mar. 2024, doi: 10.1007/s11416-024-00516-2.